1st run output:

Outputs for different data sizes

1. Changed 1024 to 128 data output:

2. Changed 128 to 256 data output:

3. Changed 256 to 512 data output:

4. Changed 512 to 1024 data output:

5. Changed 1024 to 2048 data output:

Throughput vs data size graph:

| Data Size (bytes) | Throughput (bps) |
| --- | --- |
| 128 | 69.33 |
| 256 | 126.22 |
| 512 | 240.0 |
| 1024 | 467.56 |
| 2048 | 922.67 |

Description of the graph:

The graph clearly shows a strong, positive, and near-linear correlation: as the packet's data size increases, the measured throughput also increases. This trend is caused by network overhead. Every packet sent includes fixed-size headers (like IP and UDP headers) in addition to the data. We can see this in the output:

- For the 128-byte packet, the total bytes sent: 156. So, (156 - 128 = 28 bytes of overhead).

- For the 2048-byte packet, the total bytes sent: 2076. So,  (2076 - 2048 = 28 bytes of overhead).

This 28-byte overhead is constant. When the packet size is small, this overhead makes up a large percentage of the total transmission, making it inefficient. As the packet size gets larger, the 28-byte overhead becomes a tiny fraction of the total, meaning more of the network's time is spent sending useful data. This increased efficiency results in a higher measured throughput.